Edith Cowan University

School of Engineering

ENS6160 Signals and Systems

# Lab Report 1

Spectral Analysis of Periodic Signals

Submiited by: Anandhu Kaithavalappil Binod

Student ID: 10730544

Date of Submission: 04 October 20026

Table of Contents,Figures and list of table

Aim:	2

Objectives	3

1. Introduction	3

2. Methodology	3

3. Results and Analysis	3

A. Sinusoidal Signal	3

Spectral Analysis of Sinusoidal Signal	3

Figure 1. Theoretical time-domain representation of the 1 MHz sinusoidal signal.	4

Figure 2. Theoretical magnitude spectrum of the sinusoidal signal.	5

1.Table A 1. Sinusoidal Wave – Theoretical vs Measured Values	5

Figure 3. FFT spectrum of the sinusoidal signal displayed on the oscilloscope.	6

B. Square Wave Signal	6

Figure 4. Time-domain graph of the 1 MHz square wave signal.	7

Table B. Theoretical values of the magnitude spectrum	8

Figure 5 . Magnitude spectrum of the 1 MHz square wave signal showing the fundamental and odd harmonic components.	9

Table B2: Square Wave – Theoretical vs Measured Values	9

Figure 6. Magnitude spectrum of the 1 MHz square wave based on the measured values.	10

C. Triangular Wave Signal	11

Figure 7. Time-domain graph of the 1 MHz Triangular wave signal.	11

Figure 8. Magnitude spectrum of the triangular wave based on the theoretical values.	15

.	15

Table C: Triangular Wave – Theoretical vs Measured Values	16

Figure9.Magnitude spectrum of the triangular wave based on the measured FFT values	16

Conclusion	17

References	17

# Aim:

To compare the frequency spectra of sinusoidal, square and triangular periodic signals by Fourier-series analysis and FFT measurements from oscilloscope.

# Objectives

- To analyse sinusoidal, square and triangular signals in time and frequency domain.

- To calculate their theoretical spectra with Fourier series.

- To determine the spectra from the Oscilloscope FFT and compare them with the theoretical ones.

# 1. Introduction

Periodic signals can be analysed in both the time and frequency domains. A periodic signal can be expressed as a sum of sinusoidal signals whose frequency is the fundamental frequency and other frequencies known as harmonics in the Fourier series. In this experiment a sinusoidal, square and triangular signal were generated and displayed on an oscilloscope. They were transformed into their frequency spectra using the FFT function and their measured results were compared with the results of the Fourier series.

# 2. Methodology

A signal generator was used to create sinusoidal, square and triangular signals which were viewed on a digital oscilloscope. These signals have been initially analyzed in the time domain. The FFT function of the oscilloscope was then used to display its frequency spectrum. The primary spectral components' frequencies and magnitudes were measured and matched to the theoretical values obtained from Fourier-series analysis.

# 3. Results and Analysis

# A. Sinusoidal Signal

## Spectral Analysis of Sinusoidal Signal

The theoretical sinusoidal signal has an amplitude of 1 V and a fundamental frequency of 1 MHz.

The sinusoidal signal is represented by:

xt=sin2πf0t

where the fundamental frequency is 1 × 10⁶ Hz.

The period of the signal was calculated using:

T0=1f0

T0=11×106

T0=1×10-6 s=1 μs

### Figure 1. Theoretical time-domain representation of the 1 MHz sinusoidal signal.

Fourier-Series Coefficients

For a sinusoidal signal:

xt=sin2πf0t

Therefore, the Fourier-series coefficients are:

a₀ = 0 , aₙ = 0, b₁ = 1, and bₙ = 0 for n ≥ 2.

The magnitude of the fundamental component is:

A1=a12+b12

A1=02+12

A1=1 V

Therefore, the theoretical spectrum contains a single frequency component at 1 MHz with a magnitude of 1 V.

### Figure 2. Theoretical magnitude spectrum of the sinusoidal signal.

Experimental Results – Sinusoidal Signal

The function generator was used with a sinusoidal output of 1 MHz at 2.0 Vpp for the sinusoidal signal. Waveform was displayed on oscilloscope and frequency spectrum of it was obtained by using FFT function.

### 1.Table A 1. Sinusoidal Wave – Theoretical vs Measured Values

| Spectrum Characteristic | Theoretical Value | Measured Value |
| --- | --- | --- |
| Peak amplitude | 1 V | −2.5 dBV V=10-2.520=0.75 V |
| Frequency, f | 1 × 10⁶ Hz | 1 × 10⁶ Hz |

2. Sketch of Spectrum Based on Measured Values

### Figure 3. FFT spectrum of the sinusoidal signal displayed on the oscilloscope.

3. Comparison with the Spectrum Obtained in the Post-Lab

In the theoretical and measured spectra, both contain one frequency component at 1MHz which is the theoretical value. The theoretical value was 1 V, but the measured value was 0.75 V – 2.5 dBV. The lower measured magnitude could be from practical measurement conditions and limitations used to perform the FFT measurement.

# B. Square Wave Signal

Time-Domain Graph

In  a square wave, the fundamental frequency is:

f0=1 MHz

Therefore, the period is:

T0=1f0

T0=11×106=1 μs

### Figure 4. Time-domain graph of the 1 MHz square wave signal.

Fourier Series Coefficients

For the square wave:
h=1 V

f0=1 MHz

a0=0,  an=0

bn=4hnπ,n odd0,n even

Since h=1 V

Fundamental,n=1:

b1=4π=1.273V

f1=1f0=1Mhz

Third harmonic, n=3:

b3=43π=0.424V
f3=3f0=3Mhz

Fifth harmonic, n=5:

b5=45π=0.255V

f5=5f0=5Mhz

Seventh harmonic,n=7:

b7=47π=0.182V

f7=7f0=7Mhz

Therefore,

b1=1.273V

b3=0.424V

b5=0.255V

b7=0.182V

Theoretical Magnitude Spectrum

The theoretical magnitude spectrum of the square wave includes the fundamental frequency and the odd harmonics. The magnitude is smaller with the higher harmonics and the even harmonics are zero.

### Table B. Theoretical values of the magnitude spectrum

| Harmonic | Frequency | Magnitude |
| --- | --- | --- |
| Fundamental, f1 | 1 MHz | 1.273 V |
| 3rd harmonic, f3 | 3 MHz | 0.424 V |
| 5th harmonic, f5 | 5 MHz | 0.255 V |
| 7th harmonic, f7 | 7 MHz | 0.182 V |

### Figure 5 . Magnitude spectrum of the 1 MHz square wave signal showing the fundamental and odd harmonic components.

Experimental Results – Square Wave

### Table B2: Square Wave – Theoretical vs Measured Values

| Spectrum Characteristic | Theoretical Values (from post-lab) |  | Measured Values |  |
| --- | --- | --- | --- | --- |
| Fundamental frequency, f₁ | 1.00 × 10⁶ Hz | 1.273V | 1.00 × 10⁶ | −6.25 mdBV |
|  |  |  |  | 1.413 V |
| 3ʳᵈ harmonic, f₃ | 3.00 × 10⁶ Hz | 0.424V | 2.980 × 10⁶ | −10.156 dBV |
|  |  |  |  | 0.439 V |
| 5ᵗʰ harmonic, f₅ | 5.00 × 10⁶ Hz | 0.255V | 4.980 × 10⁶ | −14.84 dBV |
|  |  |  |  | 0.256 V |
| 7ᵗʰ harmonic, f₇ | 7.00 × 10⁶ Hz | 0.182V | 6.980 × 10⁶ | −17.500 dBV |
|  |  |  |  | 0.189 V |

Magnitude Spectrum Based on Measured Values

### Figure 6. Magnitude spectrum of the 1 MHz square wave based on the measured values.

Comparison of Theoretical and Measured Results

The experimental spectrum was well matched with the theoretical spectrum. The measured centre frequencies 1.00 MHz, 2.98 MHz, 4.98 MHz and 6.98 MHz were found to be close to the theoretical centre frequencies 1 MHz, 3 MHz, 5 MHz and 7 MHz respectively. The even harmonics were not present in the spectrum as is theoretically expected.The odd harmonics were present and the even harmonics were not, as theoretically predicted. Theoretical and measured magnitudes showed slight differences, which can be attributed to practical measurement conditions and the limitation of the FFT measurement.

# C. Triangular Wave Signal

Time-domain graph

Given:

A=1 V

f1=200 kHz=200×103 Hz

The time period is:

T=1f1

T=1200×103=5×10-6 s

Therefore,T=5 μs

### Figure 7. Time-domain graph of the 1 MHz Triangular wave signal.

Fourier Series Coefficients

For the triangular wave:

Amplitude, A = 1 V

Fundamental frequency, f0 = 200 kHz

Period:

T=1f0

T=1200*103

T=5 ×10-6s

T=5 μs

Angular frequency:

ω0=2πf0

ω0=2π200×103

ω0=400000π rads

General Fourier series equation

xt=a0+n=1∞ancosnω0t+bnsinnω0t

For this triangular wave, the waveform is odd-symmetric. Therefore:

a0=0

an=0

Hence, only sine terms are present.

The Fourier coefficient for the triangular wave is:

bn=8An2π2sinnπ2

Since A = 1 V

bn=8n2π2sinnπ2

Fundamental Harmonic, n = 1

b1=812π2sinπ2

b1=8π2

b1=0.8106 V

Frequency:

f1=1×f0

f1=1×200 kHz

f1=200 kHz

Therefore, the fundamental component is:

0.8106sinω0t

Third Harmonic, n = 3

b3=832π2sin3π2

b3=-89π2

b3=-0.0901 V

Frequency:

f3=3×f0

f3=3×200 kHz

f3=600 kHz

Therefore, the third harmonic component is:

-0.0901sin3ω0t

Fifth Harmonic, n = 5

b5=852π2sin5π2

b5=825π2

b5=0.0324 V

Frequency:

f5=5×f0

f5=5×200 kHz

f5=1000 kHz

f5=1.0 MHz

Therefore, the fifth harmonic component is:

0.0324sin5ω0t

Seventh Harmonic, n = 7

b7=872π2sin7π2

b7=-849π2

b7=-0.0165 V

Frequency:

f7=7×f0

f7=7×200 kHz

f7=1400 kHz

f7=1.4 MHz

Therefore, the seventh harmonic component is:

-0.0165sin7ω0t

Final Fourier Series

xt=0.8106sinω0t-0.0901sin3ω0t+0.0324sin5ω0t-0.0165sin7ω0t+…

ω0=2π(200×10^3 )rad/s

ω0=400000π rad/s

b1=0.8106 V

b3=0.0901 V

b5=0.0324 V

b7=0.0165 V

Magnitude Spectrum – Plot and Description

### Figure 8. Magnitude spectrum of the triangular wave based on the theoretical values.

### .

Only odd harmonics are present in the magnitude spectrum of the triangular wave. The harmonics are 200 kHz, 600 kHz, 1.0 MHz and 1.4 MHz for the fundamental, the 3rd, 5th and 7th harmonics, respectively. It is observed that the amplitude of harmonics falls off steeply with the harmonic order in line with Fourier series of a triangular wave.That completes the post-lab theoretical section for the triangular wave.

### Table C: Triangular Wave – Theoretical vs Measured Values

| Spectrum Characteristic | Theoretical Values (from post-lab) |  | Measured Values |  |
| --- | --- | --- | --- | --- |
| Fundamental frequency, f₁ | 200 kHz | 0.8106 V | 200 kHz | -4.687 dBV |
|  |  |  |  | 0.583 V |
| 3ʳᵈ harmonic, f₃ | 600 kHz | 0.0901 V | 596 kHz | -23.437 dBV |
|  |  |  |  | 0.0673 V |
| 5ᵗʰ harmonic, f₅ | 1.00 MHz | 0.0324 V | 998 kHz | -32.187 dBV |
|  |  |  |  | 0.0246 V |
| 7ᵗʰ harmonic, f₇ | 1.40 MHz | 0.0165 V | 1.40 MHz | -37.8125 dBV |
|  |  |  |  | 0.0129 V |

Magnitude spectrum – Plot and description of measured FFT values.

### Figure9.Magnitude spectrum of the triangular wave based on the measured FFT values

The measured spectrum includes the fundamental and the odd harmonics at 200 kHz, 596 kHz, 998 kHz, and 1.40 MHz. As the harmonic order increases, the magnitude of the harmonic decreases quickly, as it should for a triangular form.

Comparison of Theoretical and Measured Results

The measured triangular wave spectrum revealed frequencies at about 200kHz, 596kHz, 998kHz and 1.40MHz, which are fundamental, 3rd, 5th and 7th harmonics, respectively. The measured frequencies were approximately 200 kHz, 600 kHz, 1.00 MHz and 1.40 MHz, which are the theoretical frequencies. The measured values also quickly diminished as harmonic number increased, as would be expected for a triangular wave. It is noted that the difference between the theoretical and measured magnitudes are small, which may be due to the practical measurement condition, as well as the limitations of the oscilloscope FFT function.

# Conclusion

Using Fourier-series theory and oscilloscope FFT measurements, the spectral characteristics of sinusoidal, square and triangular periodic signals were analysed. The fundamental frequency was the dominant component with the sinusoidal signal, and the fundamental and odd components were present in the square wave, but with decreasing amplitudes. The triangular wave did have odd harmonics as well, but they became much smaller as the order of the harmonic increased. The FFT frequencies measured were generally near the harmonic frequencies for all three waveforms. The differences in the magnitude between the theory and the measurement were observed to be small, and this difference can be explained by the practical measurement conditions and the limitation of the FFT measurement. Overall, the experimental results verified the expected Fourier-series behaviour of the three periodic signals.

# References

1.Lundheim, L. (2021). The role of Fourier series in mathematics and in signal theory. International Journal of Research in Undergraduate Mathematics Education, 7, 633–651. https://doi.org/10.1007/s40753-021-00134-z

2. Wang, Y., He, Z., & Zi, Y. (2013). Multiple harmonics fitting algorithms applied to periodic signals based on Hilbert–Huang transform. Journal of Sensors, 2013, Article 580152. https://doi.org/10.1155/2013/580152