Edith Cowan University

School of Engineering

ENS6160 Signals and Systems

# INTRODUCTION

### 4.1 Background

## 4. Introduction

Frequency response describes how a system responds to signals at different frequencies. In this experiment, the frequency response of a resistor-capacitor (RC) circuit was investigated in both the time and frequency domains. The circuit consisted of a 1.5 kΩ resistor and a 0.01 µF capacitor, with the output voltage measured across the capacitor. It operates as a low-pass filter, allowing low-frequency signals to pass with minimal attenuation while reducing higher-frequency signals.

The aim of this experiment was to determine the frequency response of the RC circuit and compare theoretical predictions with experimental measurements. Sinusoidal signals at 1 kHz, 10 kHz and 40 kHz were applied, and the output voltage and phase shift were measured using an oscilloscope. The amplitude and phase responses were also analysed over a frequency range of 0–60 kHz. Additionally, the circuit's response to a square-wave input was investigated to observe how different frequency components are affected by the filter.

# 5 Method

### 5.1 Equipment and Components

The following equipment and components were used during the experiment:

- Resistor – 1.5 kΩ

- Capacitor – 0.01 µF

- Function generator

- Oscilloscope

- Breadboard and jumper wires

### 5.2 Circuit Configuration

The RC circuit was connected with the resistor in series with the input and the capacitor connected between the output and ground. The output voltage was measured across the capacitor. The function generator supplied the input signal, while the oscilloscope was used to observe the input and output waveforms.

Figure 1: RC Low-Pass Filter Circuit Diagram

Figure 2: Physical RC Circuit Setup

### 5.3 Time-Domain Measurements

A sinusoidal input signal of approximately 15 Vpp was applied at frequencies of 1 kHz, 10 kHz and 40 kHz. The input and output waveforms were observed using the oscilloscope. At each frequency, the peak-to-peak voltages and phase differences were measured and recorded for comparison with theoretical values.

### 5.4 Frequency-Domain Analysis

The frequency response of the RC circuit was examined using sinusoidal input signals at 1 kHz, 10 kHz and 40 kHz. The input and output voltages were observed using the oscilloscope, and the measured values were used to calculate the magnitude response in decibels. These results were compared with the theoretical frequency response. The theoretical amplitude and phase responses were also plotted over a frequency range of 0–60 kHz using Python in Google Colab.

### 5.5 Square-Wave Measurements

A square-wave signal was applied to the RC circuit using the function generator. The input and output waveforms were observed using the oscilloscope to examine the circuit's response. The changes in the output waveform were analysed to understand how the RC low-pass filter affects the frequency components of a square-wave signal. The experimental observations were then compared with the expected theoretical behaviour.

3.RESULT

## 6. Results

### 6.1 Post-Lab A: Theoretical Analysis

#### 6.1.1 Frequency Response Derivation (A(a))

For the RC circuit, the output voltage is measured across the capacitor.

Using Kirchhoff’s voltage law:

3.1 Frequency response in time domain

At :

At :

At :

| Frequency | Magnitude |  | Phase |  |
| --- | --- | --- | --- | --- |
|  | Theoretical Value | Measured Value | Theoretical Value | Measured Value |
| 1kHz | 14.93 | 15.20 | -5.39 | -6.65 |
| 10kHz | 10.92 | 11.50 | -43.30 | -42.30 |
| 40kHz | 3.85 | 4.22 | -75.14 | -73.71 |

2. Sketch of input-output pairs

3. Filtering type and comparison of cut-off frequency (−3 dB point) between the measured and theoretical values

The RC circuit acts as a low-pass filter, as the output magnitude decreases with increasing frequency.

The theoretical cut-off frequency is:

The measured gain at 10 kHz was −2.81 dB, which is close to the theoretical −3 dB cut-off point. This indicates good agreement between the theoretical and experimental frequency responses.

3.2 Verify frequency response in frequency domain

The theoretical magnitude response was calculated using:

At :

At :

At :

The voltage magnitude in dBV was calculated using:

The measured gain was calculated using:

At :

At :

At :

| Frequency | Output Magnitude | Input Magnitude | Magnitude Gain | Theoretical Response |
| --- | --- | --- | --- | --- |
| 1kHz | 14.61 | 14.83dBV | -0.23dB | -0.0384dB |
| 10kHz | 12.18dBV | 14.99dBV | -2.81dB | -2.7606dB |
| 40kHz | 3.47dBV | 14.83dBV | -11.35dB | -11.821dB |

2. Comparison of measured results with theoretical (post-lab) values

The measured gains showed good agreement with the theoretical values, particularly at 10 kHz, where the difference was only 0.05 dB. A slightly larger difference of 0.47 dB was observed at 40 kHz, while the difference at 1 kHz was approximately 0.19 dB. Overall, the results followed the expected low-pass frequency response, with greater attenuation at higher frequencies.

3. Sketches of output spectra and description of comparison

At 1 kHz, the input and output magnitudes were almost equal, showing very little attenuation. At 10 kHz, the output magnitude decreased slightly, while at 40 kHz, it dropped significantly to 3.47 dBV. This confirms the low-pass behaviour of the RC circuit, where higher frequencies are attenuated more strongly.