Abstract.

Table of Contents.

List of Figures.

- Fig1. The graphs of the amplitude and phase response

- Fig2. The frequency when amplitude is -3dB

- Fig3. The magnitude and phase of output voltage

- Fig4. The input-output pairs

- Fig5. The comparison of input and output spectra

- Fig6. The output spectra

- Fig7. The input and output time domain waveforms

1.INTRODUCTION.

2. Method.

numbers that were measured were later changed into volts. Then checked against the theoretical value (Edith Cowan University, n.d.).

- Results.
3.1 Verify frequency response in time domain

- Vout=15*H(jω)

- amplitude ratio can be calculated by:

- |H(jω)|=11+(ωRC)2

- so if f=1kHz, ωRC=2π∗1000∗1.5∗10−5=0.09425

- |H(jω)|=11+0.094252≈0.9956

- Vtheo=15*0.9956=14.93,∠Vo=0+∠H(jω)=-5.39°

- so if f=10kHz, ωRC=2π∗10000∗1.5∗10−5=0.9425

- |H(jω)|=11+0.94252≈0.7277

- Vtheo=15*0.7277=10.92,∠Vo=0+∠H(jω)=-43.3°

- so if f=40kHz, ωRC=2π∗40000∗1.5∗10−5=3.7699

- |H(jω)|=11+3.76992≈0.2568

- Vtheo=15*0.2568=3.86,∠Vo=0+∠H(jω)=--75.1°

| - Frequency | - Magnitude |  | - Phase |  |
| --- | --- | --- | --- | --- |
|  | - Theoretical Value | - Measured Value | - Theoretical Value | - Measured Value |
| - 1kHz | - 14.93 | - 15.4 | - -5.39° | - -6.98° |
| - 10kHz | - 10.92 | - 11.4 | - -43.3° | - -42.86° |
| - 40kHz | - 3.86 | - 4.08 | - -75.1° | - -67.48° |

- Table5. Theoretical frequency response vs measured frequency response

- Sketch of input-output pairs

- Fig4. The input-output pairs

- Filtering type and comparison of cut-off frequency(-3dB point) between the measured and theoretical values.

- This RC circuit is a kind of low pass filter, which allows low frequency signals to pass through it with little diminish, but as for the high frequency signals, they are diminished sharply.

- The frequency of the circuit can be calculated by using the formula below:

- f=12πRC≈10.61kHz

3.2 Verify frequency response in frequency domain

dB cam be calculated by:

- dB=20lg|H(jω)|

so if f=1kHz: 20lg(0.9956)≈-0.038dB

if f=10kHz: 20lg(0.7277)≈-2.761dB

if f=40kHz: 20lg(0.2568)≈-11.822dB

the valid value can be calculated by:

Vvalid=Vpeak2

- the measured gain=Vout-Vin

- so, if f=1lkHz, from the figxxxxxxxxxx. the Vin=15.6V, Vout=15.4V,

- Vvalid(in)=15.62≈11.0309V, dBV(input)=20lg(11.0309)=20.85dBV

- Vvalid(out)=15.42≈10.8894V, dBV(output)=20lg(10.8894)=20.74dBV

- the measured gain=20.74-20.85=-0.11dB

- so, if f=10lkHz, from the figxxxxxxxxxx. the Vin=15.6V, Vout=11.4V,

- Vvalid(in)=15.62≈11.0309V, dBV(input)=20lg(11.0309)=20.85dBV

- Vvalid(out)=11.42≈8.061V, dBV(output)=20lg(8.061)=18.13dBV

- the measured gain=18.13-20.85=-2.72dB

- so, if f=40lkHz, from the figxxxxxxxxxx. the Vin=15.4V, Vout=4.08V,

- Vvalid(in)=15.42≈10.8894V, dBV(input)=20lg(10.8894)=20.74dBV

- Vvalid(out)=4.082≈2.885V, dBV(output)=20lg(2.885)=9.20dBV

- the measured gain=9.20-20.74=-11.54dB

| Frequency | Output Magnitude | Input Magnitude | Magnitude Gain | Theoretical Response |
| --- | --- | --- | --- | --- |
| 1kHz | 20.74dBV | 20.85dBV | -0.11dB | -0.038dB |
| 10kHz | 18.13dBV | 20.85dBV | -2.72dB | -2.761dB |
| 40kHz | 9.20dBV | 20.74dBV | -11.54dB | -11.822dB |

- Table6. Frequency response (magnitude) for sinusoids

- Comparison of measured results with theoretical (post-lab) values

The values between measured results and theoretical values were fit well. But for the f=1kHz, the value differs several times, this may cause by vibe noise, poor contact and so on.

- Sketches of output spectra and description of comparison

The graph shows below:

From the graph, at 1kHz, the input and output magnitude are almost analogous. At 10kHz, the output magnitude differs slighter than the condition of 1kHz. But at 40kHz, the output magnitude slumps. There are the characteristics of low pass filter, which allows low frequency signals to pass through it with little diminish, but as for the high frequency signals, they are diminished sharply.

- Fig5. The comparison of input and output spectra

3.3 Response to square wave input

1) The calculation of the input magnitude shows in 4.3 b) and c)

| Spectrum characteristic | Output Magnitude | Input Magnitude | Magnitude Response | Theoretical Response |
| --- | --- | --- | --- | --- |
| Fundamental | 16.4063dBV | 22.61dBV | -6.20dB | -1.96dB |
| 3rd harmonic | 6.71875dBV | 13.06dBV | -6.34dB | -7.91dB |
| 5th harmonic | 2.5dBV | 8.63dBV | -6.13dB | -11.85dB |

- Table7. Square wave theoretical vs measured harmonic response

- Sketch of output spectra

- Fig6. The output spectra

- Sketch of input and output time domain waveforms and explanation

T=1f=18000=125μs

- For the input waveform:

- Vin(t)=n=odd4Vnπsin(nω0t)

- n=1,Vin(t)=19.1V

- n=3,Vin(t)=6.37V

- n=5,Vin(t)=3.82V

- so,Vin(t)=19.1sinsin(nω0t)+6.37sinsin(3ω0t)+3.82sinsin(5ω0t)

- For the output waveform:

- V=2∗10dBV20

- f=8kHz, Vout1=9.27V

- f=24kHz, Vout2=3.07V

- f=40kHz, Vout3=1.89V

- ∠H(jω)=∠(1)−∠(1+jωRC)

- =arctan0−arctanωRC1

- =−arctan(ωRC)=θ

- for f = 8KHz ωRC=2πfRC=0.754,∠H(jω)=−arctan(0.754)≈−37.02°

- for f = 24KHz ωRC=2πfRC=2.2619,∠H(jω)=−arctan(2.2619)−66.15°

for f = 40KHz ωRC=2πfRC=3.7699,∠H(jω)=−arctan(3.7699)≈−75.14°

- so,Vout(t)=9.27∗sin(ω0t−37.02°)+6.37∗sin(ω0t−66.15°)+3.82∗sin(ω0t−75.14°)

- Fig7. The input and output time domain waveforms

- explanation:

- The first graph is input waveform and it’s similar to the square signal. The second graph is output waveform.

- There are some differences between the matlab graph and real oscilloscope for the probable reasons of vibe noise, poor contact and so on.

- Discussion.

4.1 Frequency response of an RC circuit

- Determining the frequency response, H(jω)

- RCdvo(t)dt+vo(t)=vi(t)

- using Fourier transform on both sides:

- RCjωVo(ω)+Vo(ω)=Vi(ω)

- Vo(ω)(RCjω+1)=Vi(ω)

- H(jω)=Vo(ω)Vi(ω)=11+jωRC

- Calculating|H(jω)|

- According to the formulas:

- ω=2πf

- dB=20lg|H(jω)|

- |H(jω)|=11+(ωRC)2

- ∠H(jω)=∠(1)−∠(1+jωRC)=arctan0−arctanωRC1=−arctan(ωRC)

- for f = 1KHz ω=2πf=6238.19rad/s,∠H(jω)=−arctan(0.09425)≈−5.39°

- for f = 10KHz ω=2πf=62381.85rad/s,∠H(jω)=−arctan(0.9425)−43.3°

- for f = 40KHz ω=2πf=251327.41rad/s,∠H(jω)=−arctan(3.77)≈−75.1°

| - f | - ω(rad/s) | - ¦H(jω)¦ | - ¦H(jω)¦ in dB |
| --- | --- | --- | --- |
| - 1kHz | - 6283.19 | - 0.9956 | - -0.038 |
| - 10kHz | - 62831.85 | - 0.7277 | - -2.761 |
| - 40kHz | - 251327.41 | - 0.2568 | - -11.822 |

- Table1. The calculation of H(jω)|

- Plot of amplitude and phase response

- Fig1. The graphs of the amplitude and phase response

- Questions

- What kind of filtering is performed by this RC circuit?

- This RC circuit is a kind of low pass filter, which allows low frequency signals to pass through it with little diminish, but as for the high frequency signals, they are diminished sharply.

- What does -3db equate to in terms of voltage amplitude ratio?

- According to the formula:

- dB=20lg|H(jω)|

- when the value is -3db,20lg|H(jω)|=−3, so the amplitude:|H(jω)|=10−320≈0.708

- What is the cut-off frequency(-3db point on the plot)?

- Fig2. The frequency when amplitude is -3dB

- According to the graph above, the cut-off frequency when db is -3 is 10.61kHz

- How can we determine this value from the circuit parameters?

- The frequency of the circuit can be calculated by using the formula below:

- f=12πRC≈10.61kHz

- Describe the behavior of  this system in the frequency domain and in the time domain.

- Frequency domain:

- The system is a low pass filter system, so low frequency signals pass through it with little diminishment while the high frequency signals pass through it with sharp diminishment.

- Time domain:

- Low frequency signals pass through it with little change but the signals diminish when the frequency increases.

4.2 Response of the RC circuit to an AC voltage using the frequency response

- Theoretical output voltage

the phase shift is 0.

- Vout=15*H(jω):

- so if f=1kHz, |Vo|=15*0.9956=14.93,∠Vo=0+∠H(jω)=-5.39°,

- so if f=10kHz, |Vo|=15*0.7277=10.92,∠Vo=0+∠H(jω)=-43.3°

- so if f=40kHz, |Vo|=15*0.2568=3.86,∠Vo=0+∠H(jω)=--75.1°

| - f | - ω(rad/s) | - ¦H(jω)¦ | - ∠H(jω) | - ¦Vo¦ | - ∠Vo |
| --- | --- | --- | --- | --- | --- |
| - 1kHz | - 6283.19 | - 0.9956 | - -5.39° | - 14.93 | - -5.39° |
| - 10kHz | - 62831.85 | - 0.7277 | - -43.3° | - 10.92 | - -43.3° |
| - 40kHz | - 251327.41 | - 0.2568 | - -75.1° | - 3.86 | - -75.1° |

- Table2. Theoretical output voltage

- Magnitude and phase spectra of output voltage

- Fig3. The magnitude and phase of output voltage

4.3 Response of the RC circuit to square wave using the frequency response

- Fourier Series coefficients

- Vp=15V

- f0=8kHz

- x(t)=V,  0<t<T02−V,  T02<t<T0

- a0=1T00T0x(t)dt=0

- an=2T00T0x(t)cos(nω0t)dt=0

- bn=2T00T0x(t)sin(nω0t)dt=4Vnπ(n=1,3,5,7...)

- if n is even, bn=0

- so if n=1 b1=4Vnπ=4∗151∗π≈19.10V

- if n=3, b3=4Vnπ=4∗153∗π≈6.37V

- if n=5, b5=4Vnπ=4∗155∗π≈3.82V

| - Fourier Coefficient(bn) | - Value | - Frequency |
| --- | --- | --- |
| - b1 | - 19.10V | - 8kHz |
| - b3 | - 6.37V | - 24kHz |
| - b5 | - 3.82V | - 40kHz |

- Table3. Theoretical output voltage

- and c) Magnitude of frequency response and Magnitude of harmonics

- ω1=2π*8000=50265.48rad/s

- |H1|=11+(ωRC)2=11+(50265.48∗1.5∗10−5)2≈0.798

- ω3=2π*24000=150796.45rad/s

- |H3|=11+(ωRC)2=11+(150796.45∗1.5∗10−5)2≈0.402

- ω5=2π*40000=251327.41rad/s

- |H5|=11+(ωRC)2=11+(50265.48∗1.5∗10−5)2≈0.257

- V1out=19.1*0.798≈15.24V, V1invalid=19.1/2≈13.51V, dBV=20lg(13.51)≈22.61dBV,

- V1outvalid=15.24/2≈10.78V, dBV=20lg(10.78)≈20.65dBV

- V3=6.37*0.402≈2.56V, V3invalid=6.37/2≈4.50V, dBV=20lg(4.50)≈13.06dBV,

- V3outvalid=2.56/2≈1.81V, dBV=20lg(1.81)≈5.15dBV

- V5=3.82*0.257≈0.982V, V5invalid=3.82/2≈2.70V, dBV=20lg(4.50)≈8.63dBV,

- V3outvalid=0.982/2≈0.69V, dBV=20lg(0.69)≈-3.22dBV

| - f(Hz) | - ω(rad/s) | - ¦H(jω)¦ | - Input magnitude(bn) |  | - Output magnitude |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - V | - dBV | - V | - dBV |
| - Fundamental=8kHz | - 50265.48 | - 0.798 | - 19.10 | - 22.61 | - 15.24 | - 20.65 |
| - 3rd harmonic=24kHz | - 150796.45 | - 0.402 | - 6.37 | - 13.06 | - 2.56 | - 5.15 |
| - 5th harmonic=40kHz | - 251327.41 | - 0.257 | - 3.82 | - 8.63 | - 0.982 | - -3.22 |

- Table4. Magnitude of frequency response and Magnitude of harmonics

- Description of output waveform

- This low pass filter will diminish high frequency signals. According to the values, the fundamental component passes through with little deminishment while the 3rd and 5th signals are diminished heavily.

- 5. Conclusion.

- 6.REFERENCES.

- 7.Group Work Declaration.

- 8.Appendix

- Fig8. Measured value for f=1kHz

- Fig9. Measured value for f=10kHz

- Fig10. Measured value for f=40kHz

- Fig11. Measured value for f=8kHz

- Fig12. Measured value for f=24kHz

- Fig13. Measured value for f=40kHz